Hardware Components Core Processing Node

- ESP32 Development Board: The main processing unit handling analog-to-digital conversions and TLS 1.2 encryption.

Sensors s Display

- DC Voltage Sensor: Wired in parallel to measure the voltage potential of the load circuit.

- ACS712 Current Sensor: Wired in series to measure the physical current drawn.

- 16x2 LCD Display with I2C Backpack: Provides local, real-time data viewing via the D21 and D22 pins.

The Isolated Load Circuit

- DC Battery: The physical power source driving the isolated high-voltage loop.

- DC Fan: The physical high-voltage load drawing power from the battery.

Power s Prototyping

- DC005 Barrel Jack: Provides the isolated 5V power and ground to the ESP32 and sensor logic side.

- Standard Breadboard: The physical mounting platform for your logic components.

- Jumper Wires: Connects the logic rails and bridges the high-voltage paths.

Infrastructure s Networking

- POCO X7 Pro Smartphone: Acts as your physical Middle Node, providing the isolated local area network hotspot.

- Windows Laptop: The physical Infrastructure Node hosting your Mosquitto broker and Möbius database.

Software Components

- Custom C++ Firmware: The application logic flashed to the ESP32. It handles

sensor math, Wi-Fi connectivity, secure MQTT packet publishing, and oneM2M JSON payload structuring.

- Möbius Resource Browser: A Node.js web application (app.js) used as a graphical user interface to monitor and visualize the oneM2M resource tree in real-time.

- OpenSSL Certificates: The ca.crt, server.crt, and server.key files required to enable TLS 1.2 encryption on the Mosquitto broker.

Platforms and Services Used Platforms s Servers

- Möbius oneM2M Platform: An enterprise-grade, open-source IoT server acting as the Common Services Entity (CSE). It manages access control, authenticates

identity headers (SOrigin), and securely stores the sensor data in nested containers.

- Eclipse Mosquitto: An open-source message broker implementing the MQTT protocol. Configured as a dual-listener bridge to accept external encrypted MQTTS traffic (Port 8883) and route it to the internal Möbius database (Port 1883).

- Blynk IoT Platform: A cloud-based presentation layer platform used to build the mobile dashboard and display live energy metrics to the end user.

Development s Testing Tools

- Arduino IDE: The primary integrated development environment used to write, compile, and upload the C++ firmware to the ESP32 edge node.

- Postman: An API development tool used extensively to provision the oneM2M database structure (POST requests) and verify secure data retrieval (GET requests) using strict HTTP headers.

- Wireshark: A network protocol analyzer used  during the testing phase to capture live network traffic and definitively prove the successful implementation of TLS 1.2 ciphertext encryption.